# Analytic Trigonometry Unit 07 Readings:

# Complex Numbers

remember:

≡  means "is defined to be"

Complex numbers

"imaginary" unit i ≡ -1

complex numbers have a real and an imaginary part: a + bi

every real number is a complex number in which the imaginary part equals zero

usually designated "z"

Rules for complex numbers:

equality:		a + bi  = c + di        if   a=c   and   b=d

addition:		(a + bi ) + (c + di) = (a + c) + (b + d) i

subtraction:	(a + bi ) – (c + di) = (a – c) + (b – d) i

complex conjugate:	(a + bi ) × (a – bi) = a 2+ b 2

For -n  ALWAYS TAKE OUT THE “i ” first!

-4  =  i 4  =  2 i

-9 × -36   = 3i × 6i = 18i2 = –18 (the right answer)

If you don’t take out the “i ” first:

-9 × -36   = 324 = 18 (the wrong answer)

### The Complex Plane

imaginary unit i = -1

i = -1

i 2 = –1

i 3 = – i

i 4 = 1

i 5 = i

.

.

.

complex numbers can be plotted

as points on the complex plane

2+3i

real axis (horizontal) and an

imaginary axis (vertical)

–2+i

Graphing Complex Numbers

We call the graph paper the "complex plane"

Given a complex number, to represent its components:

1.5–2.5i

Move along the horizontal axis the amount of the real part

Move parallel to the vertical axis the amount of the imaginary part

Plot the point

Complex Numbers in Electronics

A common situation in electrical networks is the existence of many sine-shaped waves all with

the same frequency but with different amplitudes and phases

They are represented by a real multiplier for frequency and time and a complex multiplier for

amplitude (phasor)

Combining these phasors is called phasor arithmetic

In AC circuits, resistors, capacitors, and inductors can be combined in a single complex

number called the impedance (in ohms, Ω):

Z = R + j·X

R is the resistance (a real number)

X is the reactance (imaginary)

These are generally written:

Z = (17 + j·1.5)Ω

The imaginary unit, i, is represented by the letter "j", to avoid confusion with current (Ɪ )

If two impedances are connected in series, the equivalent impedance is obtained by addition:

Ze= Z1 + Z2

The total impedance zT in a parallel circuit composed of circuits z1 and z2 is given by the formula: 					    zT = z1z2z1+z2

Kirchhoff's circuit laws (you will be running into them in your electrical engineering courses) work with phasors in complex form